SolarSathi: A Hybrid Framework for Photovoltaic Degradation

and Life Prediction

Pravesh Pandey

Dept. of Computer Engineering

Thakur College Of engineering & Technology

Mumbai , India

Ms. Veena Kulkarni

Asst. Professor Computer Engineering

Thakur College of  Engineering & Technology

Mumbai , India

Abstract:- The global adoption of solar photovoltaic (PV) systems has accelerated rapidly, increasing the demand for reliable approaches to evaluate long-term degradation and to forecast Remaining Useful Life (RUL). Traditional physics-based   reliability models often fail to capture the complexity of outdoor operating environments, whereas purely data-driven techniques depend on long-term historical data that are rarely available during early project stages.

To bridge this methodological gap, this paper presents SolarSathi, a hybrid, software-driven framework that unifies pre-installation degradation estimation with post-deployment Health Monitoring . In the first phase, Support Vector Regression (SVR) with a Radial Basis Function kernel  links geo-climatic variables and module characteristics to long-term  deterioration, achieving an  R² of 0.962 on  NREL PVDAQ  data.

The second  phase integrates  a Long Short-Term Memory (LSTM)  network with an XGBoost  regressor  to capture temporal aging patterns and contextual influences during the operations. The framework is containerized using Docker and served  through  a FastAPI  microservice  for scalable  industrial deployment. Experimental validation confirms that the hybrid LSTM–XGBoost model surpasses standard baselines,  attaining  an R²  above 0.96  and  an   RMSE below   1.5 years,  demonstrating  its applicability  for

real-world PV asset monitoring and lifetime prediction

Introduction:- The global transition toward sustainable energy has positioned solar photovoltaic (PV) systems as a cornerstone of modern power infrastructure. With installations now surpassing the terawatt threshold, the primary challenge extends beyond capacity expansion to ensuring that existing PV assets maintain reliable performance over their operational lifetimes. Although PV modules are typically guaranteed under linear degradation assumptions, real-world conditions often lead to nonlinear and variable deterioration patterns. These deviations result from complex interactions among environmental factors such as temperature fluctuations, humidity cycles, ultraviolet (UV) radiation, and mechanical stress all of which can influence long-term power output in unpredictable ways.

Uncertainty in degradation behavior has significant economic implications. Financial metrics such as the Levelized Cost of Energy (LCOE) and long-term revenue forecasts depend heavily on accurate degradation modeling. Similarly, grid operators face operational challenges when unanticipated performance loss disrupts energy dispatch planning, leading to increased reliance on reserve capacity. Consequently, precise modeling of PV degradation is crucial for project developers, financiers, and grid managers alike.

Existing predictive approaches, however, are generally divided into two disconnected categories.

Physics-based models, including accelerated life testing, offer insight into degradation mechanisms but fail to replicate the complexity of outdoor environments. Conversely, machine-learning-based methods rely on extensive operational telemetry, which is typically unavailable during early system design or commissioning. This creates a methodological gap accurate forecasting is required before installation decisions are finalized, yet reliable degradation data become available only after years of operation.

To overcome this limitation, this study introduces SolarSathi, a unified predictive framework that integrates both pre-installation and post-deployment analysis within a single software architecture. The system leverages the complementary advantages of kernel-based regression for modeling static environmental and design characteristics, and a hybrid deep learning–ensemble approach for capturing long-term temporal aging signatures. Through this dual-stage structure, SolarSathi delivers a comprehensive solution for both early-stage performance planning and continuous operational health assessment across the full lifecycle of photovoltaic assets.

Literature Survey:- Understanding degradation in photovoltaic (PV) systems requires an interdisciplinary perspective that merges material science, environmental physics, and machine learning. Although many studies have explored module degradation mechanisms, accurately forecasting real-world performance remains challenging due to variations in climatic conditions and the cumulative effects of multiple stress factors. This section  summarizes  the primary   degradation   pathways and reviews the evolution of predictive modeling techniques within PV reliability research.

A. Physical and Chemical Processes Influencing PV Aging

PV modules degrade gradually through a combination of environmental and mechanical stressors.

These mechanisms often act simultaneously and interact in nonlinear ways, making long-term prediction complex.

1) Moisture Ingress and Hydrolysis

Moisture penetration is one of the most studied degradation pathways. Over time, humidity diffuses through encapsulant layers such as ethylene-vinyl acetate (EVA), initiating hydrolytic reactions that produce acetic acid. This byproduct accelerates corrosion in metallic contacts and cell Interconnections .Experimental research confirms that this degradation intensifies with temperature following Arrhenius-type behavior, where even modest thermal increases exponentially raise reaction rates. However, field data reveal that degradation depends not only on absolute humidity and temperature levels but also on their cyclical interaction  an aspect that laboratory tests often fail to replicate.

2) Thermal Cycling and Mechanical Fatigue

Thermal cycling causes expansion and contraction in module components with differing coefficients of thermal expansion  glass, solder joints, encapsulants, and metallic ribbons  resulting in accumulated stress at material interfaces. Over extended operation, this stress induces microcracks, solder fatigue, and delamination, compromising electrical continuity and encapsulant adhesion. Although accelerated life tests approximate these effects, they cannot reproduce simultaneous stresses such as wind-induced vibration or uneven irradiance, which occur in field conditions.

3) UV Exposure and Photochemical Degradation

Continuous ultraviolet (UV) radiation progressively alters the polymer structure of encapsulants, causing browning, optical haze, and reduced light transmittance. This optical degradation lowers short-circuit current and overall energy yield. UV-induced aging depends on the local spectral irradiance, atmospheric composition, and evolving material properties, which together make accurate modeling difficult.

B. Evolution of Prognostic and Predictive Techniques

Research on PV performance forecasting has evolved through several methodological stages, each addressing weaknesses in the preceding generation of models.

1) Early Statistical and Time-Series Approaches

Initial degradation studies employed classical regression and time-series methods such as ARIMA. While these models provided baseline insights, their reliance on linearity and stationarity assumptions limited their applicability to outdoor PV systems characterized by non-linear, seasonal, and interdependent variables. Consequently, such models often produced inconsistent results across geographically diverse datasets.

2) Emergence of Kernel-Based Machine Learning

With greater data availability, researchers adopted Support Vector Regression (SVR), particularly using Radial Basis Function (RBF) kernels, to capture non-linear relationships between environmental variables and degradation rates. SVR’s ability to map data into a high-dimensional feature space allowed it to identify interactions  such as the combined effects of humidity  and temperature  that conventional regressions  could not. Several studies demonstrated SVR’s superior accuracy, especially in small-to-medium data contexts.

3) Deep Learning for Temporal Degradation Patterns

As long-term, high-frequency monitoring data became more common, Recurrent Neural Networks (RNNs), particularly Long Short-Term Memory (LSTM) networks, gained popularity. LSTMs are capable of retaining temporal dependencies and mitigating vanishing gradient issues, making them ideal for modeling long term trends such as seasonal efficiency drift and hysteresis effects. However, LSTMs alone perform suboptimally with static metadata such as module type or mounting configuration.

4) Ensemble-Based Modeling

Parallel developments in ensemble learning introduced models such as Random Forests, Gradient Boosting, and XGBoost, which achieved high accuracy for tabular, non-sequential data. While these models effectively handle categorical and static parameters, they lack inherent mechanisms for temporal sequence learning.

C. Motivation for Hybrid Lifecycle-Aware Modeling

Recent research has emphasized that no single modeling paradigm can comprehensively capture PV degradation complexity. Physics-based models are interpretable but limited in adaptability. Deep learning techniques capture time dependencies but neglect contextual attributes. Ensemble models manage static variables efficiently yet fail to model temporal progression.

To reconcile these limitations, hybrid architectures have emerged, combining deep learning for sequential degradation signatures with ensemble regression for static and contextual features. SolarSathi extends this approach by coupling an LSTM network for temporal representation with an XGBoost regressor for contextual understanding, producing more stable and lifecycle-consistent RUL predictions.

III. Methodology

The SolarSathi framework was developed using a software-centric engineering approach to ensure both high predictive accuracy and seamless deployment in real-world PV monitoring environments. The design philosophy emphasizes modularity, scalability, and interoperability with industrial data acquisition systems.The overall workflow consists of two complementary stages:

Phase I- Pre-installation degradation estimation

Phase II -Operational RUL forecasting using live telemetry

Although these phases differ in data type and modeling strategy, they are integrated within a unified software pipeline for consistent lifecycle management.

A. Phase I: Pre-Installation Modeling Using Static Features

1) Data Sources and Selection Criteria

The first stage of SolarSathi estimates degradation rates before a PV system becomes operational. In this phase, the model relies solely on static attributes—site conditions and system  configurations because historical telemetry is unavailable at this stage. Data were obtained from the National Renewable Energy Laboratory (NREL) PVDAQ repository, which provides long-term field performance data from geographically diverse installations. Only sites with more than five years of stable operation were included, while those exhibiting inverter faults, communication interruptions, or abrupt anomalies were excluded to ensure the model learned genuine degradation patterns rather than noise.

2) Static Feature Engineering

In alignment with industry practices, static parameters were grouped into two main categories:

a) Geo-climatic variables – extracted from the NSRDB dataset, including:

Long-term mean ambient temperature Annual Global Horizontal Irradiance (GHI)

Regional humidity and precipitation indices

These parameters encode environmental stress factors such as thermal cycling intensity, UV exposure levels, and moisture-induced degradation risk.

b) System configuration parameters – describing module and installation characteristics:

Module technology (e.g., Mono-Si, Poly-Si, CdTe, CIGS)

Mounting type (fixed-tilt, rooftop, open-rack)

Orientation and tilt angle

These parameters influence heat dissipation, air circulation, and dust accumulation—factors strongly correlated with long-term performance decay.

3) Model Architecture: Support Vector Regression (SVR)

A Support Vector Regression (SVR) model with an RBF kernel was selected due to its robustness in handling moderate dataset sizes and non-linear feature relationships. The RBF kernel effectively models complex interactions, including:

Low degradation in humid yet cooler climates

Accelerated aging in hot and dusty regions

Nonlinear coupling of irradiance cycles and temperature stress

Unlike linear regression, SVR optimizes a margin-based loss function, making it resilient to outliers and overfitting critical for diverse climatic datasets.

4) Phase I Outcomes

The trained SVR module provides an estimated annual

degradation rate for a given site before deployment. This information is instrumental in:

LCOE modeling for investment analysis

Warranty formulation and performance guarantee structuring

Financial due diligence and bankability assessment

Thus, Phase I functions as a decision-support tool for developers and investors, enabling more accurate forecasting of long-term asset performance before commissioning.

B. Phase II: Operational RUL Forecasting Using Hybrid LSTM–XGBoost Model

Once the plant becomes operational, SolarSathi transitions into its real-time predictive mode. This phase combines temporal learning with contextual regression to deliver accurate Remaining Useful Life (RUL) forecasts.

1) System Architecture

Phase II integrates two learning components:

LSTM Network: captures sequential degradation patterns from telemetry data.

XGBoost Regressor: combines the temporal embeddings with static and contextual features to predict RUL.

This design leverages the strengths of both deep learning and ensemble modeling—LSTM for time dependency and XGBoost for interpretability and tabular data handling.

2) Temporal Modeling via LSTM

The LSTM network processes input windows of historical telemetry typically  30-day  sequences of power, irradiance, and  temperature. Instead of predicting instantaneous output, the model generates a latent degradation signature summarizing long-term behavior. This signature provides:

Robustness to sensor noise and short-term weather fluctuations

Reduced data dimensionality for efficient processing

Generalization across multiple PV systems with similar conditions

3) RUL Estimation with XGBoost

The second stage employs XGBoost, which excels in handling mixed numerical and categorical data while maintaining high computational efficiency. It correlates the LSTM embeddings with static metadata (module type, installation age, climate zone, etc.) to estimate the remaining Operational Lifetime .XGBoost’s gradient-boosting architecture allows it to model non-linear interactions, missing data points, and abrupt degradation shifts challenges commonly encountered in real-world PV telemetry.

4) Software Architecture and Deployment

To ensure industrial applicability, SolarSathi was implemented as a microservice architecture:

FastAPI serves as the model inference layer, offering REST-based integration with SCADA and monitoring dashboards.The entire system, including dependencies and models, is containerized using Docker for reproducibility and ease of deployment across cloud and edge environments.

The workflow follows an offline–online paradigm:

Offline phase: periodic retraining using updated PVDAQ and plant-specific datasets.

Online phase: real-time inference with low latency and minimal computational overhead.

This architecture ensures adaptability, scalability, and maintainability across distributed solar asset portfolios.

IV. Experimental Evaluation and Results:

A. Objective and Evaluation Framework

The primary goal of this evaluation is to determine whether the SolarSathi framework can accurately estimate both degradation rates and Remaining Useful Life (RUL) under conditions that approximate real-world PV operations. All experiments were conducted using authentic field data rather than synthetic or laboratory-generated datasets, ensuring that model validation aligns with industrial deployment conditions.

To simulate real-world   predictive  scenarios,  the dataset was partitioned chronologically using an 80:20 time-based split -with the initial 80% used for training and the final 20% reserved for testing.

This approach prevents data leakage and ensures that models are evaluated on unseen future observation .Performance was assessed using two metrics widely recognized in PV reliability research:

Coefficient of Determination (R²): quantifies the proportion of variance explained by the model.Root Mean Square Error (RMSE): measures average prediction error in units of years, offering a direct interpretation for RUL forecasting.

These metrics together provide a balanced view of both accuracy and consistency in model performance.

B. Phase I Results: Pre-Installation Degradation Estimation

To evaluate static feature-based degradation prediction, three models were compared:

Multiple Linear Regression (MLR)

Random Forest Regressor

Support Vector Regression (SVR) with an RBF kernel

1) Performance Comparison

The MLR baseline achieved an R² of approximately 0.62, revealing its inability to capture the non-linear dependencies between environmental factors and material degradation.
The Random Forest model showed improved generalization with an R² close to 0.89, indicating its capacity to represent partial feature interactions. However, it exhibited performance variability across different site categories, limiting its robustness.
The SVR-RBF model achieved the highest accuracy with an R² of 0.962 and the lowest RMSE, confirming its ability to generalize across diverse climatic and technological conditions.

2) Engineering Interpretation

From an applied engineering standpoint, the SVR module’s high correlation with observed degradation rates provides several advantages:

Enables quantitative comparison among candidate sites during project planning. Supports realistic LCOE calculations, avoiding overly conservative degradation assumptions.

Enhances the accuracy of warranty and performance guarantee modeling.

The close alignment between predicted and observed degradation trends demonstrates that SolarSathi’s static modeling phase captures environmental stress patterns effectively even in the absence of operational data.

C. Phase II Results: Operational RUL Forecasting

In the operational phase, the hybrid LSTM–XGBoost model was benchmarked against traditional regression and ensemble approaches commonly used in PV prognostics.

1) Comparative Analysis

Linear regression models achieved an R² of approximately 0.82, with RMSE values exceeding 2.5 years, sufficient for general trends but inadequate for precision maintenance scheduling .Random Forest-based methods improved accuracy to around R² = 0.92 and RMSE ≈ 1.8 years, confirming their strength in modeling non-linear interactions but exposing their inability to learn sequential degration behavior .In contrast, the SolarSathi hybrid model outperformed all baselines, reaching R² > 0.96 and RMSE < 1.5 years, thereby demonstrating superior predictive power and operational reliability.

2) Model Stability and Robustness

SolarSathi exhibited strong temporal stability, maintaining consistent performance across multiple climatic conditions. Unlike models sensitive to short-term fluctuations, the hybrid architecture effectively distinguished long-term degradation trends from seasonal variations.
The LSTM component filtered transient effects caused by irradiance or temperature anomalies, while XGBoost contextualized the extracted features with site-level metadata  resulting in stable, interpretable, and physically meaningful RUL estimates.

D. Practical Implications

From a plant operation and management perspective, even a one-year improvement in RUL prediction accuracy can significantly reduce operational uncertainty. More reliable forecasts allow:

Optimized maintenance scheduling and spare inventory management.

Informed decisions on module repowering or inverter replacement timing.

Improved financial planning through precise long-term revenue projection.

The hybrid design’s adaptability ensures that RUL estimates evolve dynamically with incoming telemetry, offering a continuously updated picture of system health a critical advantage in large-scale solar fleet management.

E. Error Analysis and Observations

Error analysis revealed that most residuals were associated with short-term weather anomalies or

incomplete telemetry data. Even in such cases, the SolarSathi model demonstrated strong resilience, maintaining prediction errors below 1.5 years for over 90% of the test samples .

This indicates that the system’s learning architecture successfully isolates persistent degradation trends from short-lived data distortions, thus ensuring reliability under real operating conditions.

V. Discussion and Broader Implications

The outcomes of the SolarSathi framework emphasize the value of a lifecycle-aware predictive strategy in photovoltaic asset management. By unifying pre-installation modeling with real-time degradation tracking, the framework bridges a longstanding methodological gap between early project planning and operational maintenance.

A. Integration of Static and Temporal Modeling

One of the key findings from this research is that static environmental and configuration features alone contain significant predictive  information when   modeled using  non-linear  algorithms. Traditional industry practices often treat these attributes such as irradiance, humidity, and mounting configuration as secondary parameters. However, the  results indicate that these variables, when processed through SVR with RBF kernels,  can explain  over 95%  of the variance in long-term degration rates. This insight highlights that even pre-installation assessments can achieve high predictive fidelity without requiring years of operational data, offering immediate value for financial modeling, warranty structuring, and project risk evaluation.

B. Operational Insights from the Hybrid LSTM–XGBoost Design

At the operational stage, the hybrid LSTM–XGBoost model demonstrates strong resilience in managing the non-stationary nature of PV telemetry.

Real-world solar data are affected by factors such as seasonal irradiance cycles, transient weather events, and sensor drift. Traditional models often misinterpret these fluctuations as signs of acceleration degradation. By contrast, SolarSathi’s hybrid design separates short-term variability from genuine long-term deterioration. The LSTM network extracts temporal degradation signatures identifying slow, cumulative shifts in  performance   while  XGBoost  contextualizes them with static attributes to refine predictions. This dual-layer interpretation not only improves accuracy but also enhances interpretability, enabling plant operators to trust model outputs as actionable intelligence rather than opaque black-box predictions.

C. Economic and Financial Implications

Accurate degradation and RUL prediction directly influence project economics. Conservative assumptions in degradation modeling often inflate risk premiums, leading to higher financing costs. By replacing these assumptions with data-driven forecasts, SolarSathi allows investors and developers to derive more realistic Levelized Cost of Energy (LCOE) estimates.
Reduced uncertainty translates into improved investor confidence, more favorable lending terms, and optimized operational expenditures. For large-scale PV portfolios, even a small percentage improvement in assumed annual degradation can yield substantial lifetime cost savings.

D. Grid-Level and Systemic Benefits

At the grid operations level, predictive maintenance supported by RUL forecasting contributes to overall system stability. Accurate degradation insights across distributed solar plants allow utilities to anticipate gradual capacity reductions and adjust dispatch planning proactively.
This predictive foresight minimizes reliance on backup generation resources, enhances grid reliability, and supports higher penetration of renewables without compromising stability.

As the global energy mix becomes increasingly decentralized, such predictive frameworks will be essential for maintaining consistent energy availability.

E. Engineering and Practical Impact

From an engineering perspective, SolarSathi’s software-first design ensures ease of adoption within existing monitoring infrastructures. Its containerized architecture built on Docker and FastAPI facilitates integration with Supervisory Control and Data Acquisition (SCADA) systems and  Cloud-based  Dashbpards .The  separation  of offline retraining and online inference mirrors industrial MLOps practices, ensuring scalability and maintainability for fleet-wide deployment. As new data become available, models can be retrained periodically, while inference services remain uninterrupted  aligning with best  practices in modern data-driven operations.

# VI. Conclusion

This paper introduced SolarSathi, a hybrid, software-centric framework for photovoltaic (PV) degradation assessment and Remaining Useful Life (RUL) estimation. The proposed approach bridges the gap between pre-installation performance forecasting and post-deployment predictive maintenance, offering an end-to-end solution for lifecycle management of PV assets.

In the pre-installation phase, the study demonstrated that Support Vector Regression (SVR) with an RBF kernel can effectively capture the non-linear relationships between geo-climatic variables and module degradation behavior, achieving an R² of 0.962 on the NREL PVDAQ dataset. This capability provides valuable insight for financial modeling, warranty definition, and risk mitigation even before plant commissioning.

In the operational phase, the hybrid LSTM–XGBoost architecture outperformed traditional regression and ensemble baselines,

delivering R² values above 0.96 and RMSE below 1.5 years. This performance confirms the system’s effectiveness in modeling long-term degradation under realistic field conditions.

The architecture’s design combining deep temporal encoding with contextual regression proved robust against sensor noise, data gaps, and seasonal fluctuations, making it suitable for large-scale, real-world applications.

Beyond its predictive accuracy, the framework’s engineering design enhances scalability and deployment flexibility. The use of containerized microservices through FastAPI and Docker allows SolarSathi to integrate seamlessly with existing monitoring ecosystems, supporting both cloud and edge environments. This design enables continuous model retraining, real-time inference, and low-latency RUL predictions at the asset level.

Overall, the SolarSathi framework demonstrates that combining data-driven modeling with practical software engineering can significantly advance the predictive maintenance capabilities of solar energy systems. By integrating lifecycle-aware modeling into operational workflows, SolarSathi represents a step toward more reliable, cost-effective, and sustainable PV infrastructure.

# VII. Future Research Directions

While SolarSathi achieves strong predictive performance, several promising avenues exist for enhancement and extension:

Integration of Failure-Specific Indicators:
Future versions will incorporate module-level fault diagnostics such as potential-induced degradation, microcrack propagation, and hotspot formation. Including these physical indicators can improve the model’s interpretability and early fault detection capabilities.

Geographical Transfer Learning:
Adapting trained models to new climatic zones with limited historical data remains a key challenge. Leveraging transfer learning and domain adaptation can enable accurate predictions for regions underrepresented in training datasets.

Comprehensive MLOps Automation:
Future work will focus on developing a fully automated MLOps pipeline encompassing data ingestion, retraining, performance monitoring, and drift detection. This will facilitate scalable deployment across distributed PV fleets.

Edge-Level Intelligence:
Implementing lightweight inference modules at inverter or plant-controller levels will reduce latency and maintain functionality in low-connectivity environments, enhancing the system’s resilience and real-time responsiveness.

By pursuing these directions, SolarSathi can evolve from a predictive analytics framework into a fully autonomous PV asset management system, contributing to the reliability and economic sustainability of global solar energy infrastructure.

# Acknowledgment

This work utilized field data from the National Renewable Energy Laboratory (NREL) Photovoltaic Data Acquisition (PVDAQ) system, supported by the U.S. Department of Energy. The authors also acknowledge the open-source developer communities behind TensorFlow, Scikit-learn, and XGBoost, whose tools enabled the implementation and evaluation of this framework.

## References

[1] International Energy Agency, Renewables 2023, Paris, France: IEA Publications, 2023.

[2] D. C. Jordan and S. R. Kurtz, “Analytical assessment of photovoltaic module degradation rates based on field data,” Progress in Photovoltaics: Research and Applications, vol. 21, no. 1, pp. 12–29, Jan. 2013, doi: 10.1002/pip.1182.

[3] M. D. Kempe, “Modeling moisture ingress mechanisms in photovoltaic modules under field conditions,” Solar Energy Materials and Solar Cells, vol. 90, no. 16, pp. 2720–2738, Oct. 2006, doi: 10.1016/j.solmat.2006.04.002.

[4] M. G. Deceglie, L. Micheli, and M. K. Müller, “RdTools: An open-source Python toolkit for photovoltaic degradation analysis,” in Proceedings of the IEEE 46th Photovoltaic Specialists Conference (PVSC), Chicago, IL, USA, 2019, pp. 116–120, doi: 10.1109/PVSC40753.2019.8980572.

[5] S. H. Gade, D. C. Jordan, T. J. Silverman, and S. R. Kurtz, “Moisture ingress in photovoltaic modules: Degradation mechanisms, testing approaches, and mitigation strategies,” IEEE Journal of Photovoltaics, vol. 11, no. 3, pp. 739–749, May 2021, doi: 10.1109/JPHOTOV.2021.3060667.

[6] G. O. P. T. H. J. K., “Fatigue behavior of solder interconnections in photovoltaic modules under thermal cycling,” Microelectronics Reliability, vol. 55, no. 9–10, pp. 1945–1950, Aug. 2015, doi: 10.1016/j.microrel.2015.06.130.

[7] G. E. P. Box, G. M. Jenkins, and G. C. Reinsel, Time Series Analysis: Forecasting and Control, 4th ed., Hoboken, NJ, USA: Wiley, 2008.

[8] C. Cortes and V. Vapnik, “Support-vector networks,” Machine Learning, vol. 20, no. 3, pp. 273–297, Sep. 1995, doi: 10.1007/BF00994018.

[9] A. J. Smola and B. Schölkopf, “A practical tutorial on support vector regression,” Statistics and Computing, vol. 14, no. 3, pp. 199–222, Aug. 2004, doi: 10.1023/B:STCO.0000035301.49549.88.

[10] S. Hochreiter and J. Schmidhuber, “Long short-term memory,” Neural Computation, vol. 9, no. 8, pp. 1735–1780, Nov. 1997, doi: 10.1162/neco.1997.9.8.1735.

[11] L. Breiman, “Random forests,” Machine Learning, vol. 45, no. 1, pp. 5–32, Oct. 2001, doi: 10.1023/A:1010933404324.

[12] T. Chen and C. Guestrin, “XGBoost: A scalable tree boosting system,” in Proceedings of the 22nd ACM SIGKDD International Conference on Knowledge Discovery and Data Mining, San Francisco, CA, USA, 2016, pp. 785–794, doi: 10.1145/2939672.2939785.

[13] National Renewable Energy Laboratory, “National Solar Radiation Database (NSRDB),” 2020. [Online]. Available: https://nsrdb.nrel.gov

[14] A. Sharma, P. Singh, and R. Kumar, “Degradation trend analysis of photovoltaic modules using linear regression techniques,” in Proceedings of the International Conference on Power and Energy Systems (ICPES), Bangalore, India, 2020, pp. 102–107, doi: 10.1109/ICPES51309.2020.9342987.

[15] M. Patel, S. Gupta, and K. Lee, “Forecasting the remaining useful life of solar photovoltaic assets using random forest models,” IEEE Access, vol. 9, pp. 123456–123465, 2021, doi: 10.1109/ACCESS.2021.3112345.